Space Object Crossing and Visibility Analysis Using TLE-Based Orbit Propagation

Author: Upasana Panigrahi

- Introduction

The rapid increase in the number of active satellites, debris fragments, and operational spacecraft in Earth orbit has significantly increased the importance of reliable space object detection and tracking systems. Space Situational Awareness (SSA) systems rely on networks of ground-based and space-based sensors to monitor objects in orbit, predict their trajectories, and ensure the safety and sustainability of the space environment. A fundamental component of such systems is the ability to determine when a space object enters the field of view of a sensor and whether the object is detectable under operational constraints.

This project presents a computational framework for analyzing crossing and visibility events between a sensor platform and an orbiting space object using publicly available Two-Line Element (TLE) data. The analysis uses the SGP4 orbit propagator to determine the position of the target object, while the sensor platform is modeled using a simplified two-body Keplerian orbit propagation. The sensor is assumed to possess a fixed field-of-view camera whose boresight is aligned with the velocity vector of the tracking platform.

The objective of the analysis is twofold. First, the algorithm determines geometric crossing events in which the target object enters the sensor’s field of view. Second, the algorithm evaluates visibility conditions to determine whether the object is detectable during those crossings based on illumination and range constraints. The analysis is performed over a 24-hour time window using a discretized time grid.

In addition to the single-object analysis, the project proposes a scalable computational architecture capable of performing similar analyses for tens of thousands of space objects and multiple sensors. Such scalability is essential for modern SSA operations, where large object catalogs must be screened continuously to support space safety, collision avoidance, and mission planning.

- Assumptions

- Target propagation:
The target object is propagated with the SGP4 model using the given TLE, yielding position and velocity in the TEME frame. It is assumed that the TLE is valid during the entire duration of the analysis, which is 24 hours.

- Tracker propagation:
The space-based tracker is propagated by a two-body Keplerian model assuming a circular orbit, starting from the given orbital elements. Higher-order perturbations like J2, atmospheric drag, and third-body effects are all ignored, which is a reasonable approach for the purpose of short-term geometric screening.

- Coordinate frame handling:
Relative geometry is determined by using the SGP4 TEME position vectors. During this 24-hour analysis, TEME is treated as an inertial-like frame and is free of any TEME ↔ GCRF/ITRF transformations, Earth rotation, or polar motion corrections.

- Sensor field-of-view model:
An ideal conical FOV with a full angle of 30° (half-angle 15°) is used to model the camera's field of view.

- Sensor pointing law:
The sensor's boresight is assumed to be pointing directly along the instantaneous velocity vector of the tracker. Factors such as misalignment, jitter, slew limits, or attitude constraints are all neglected.

- Crossing definition (geometry only):
A crossing event happens when the angle between the line-of-sight from the tracker to the target and the sensor's boresight is at most 15°. This definition is based solely on geometry and does not take into account factors like detection probability, SNR, PSF, or exposure time.

- Detectability rule (space-based):
A visible/detectable event is defined by the following binary condition:
crossing AND target sunlit AND tracker–target range < 1000 km, with a strict range threshold.

- Illumination model:
The illumination of the target is assessed with the help of a low-precision analytical Sun position and a cylindrical Earth shadow model. Effects of the penumbra and atmospheric refraction are not considered.

- Time discretization:
The analysis is performed with a constant time interval Δt (for instance, 10 seconds). Consequently, event start and end times are determined to within about ±Δt, and no root-finding or interpolation techniques are used.

- Operational simplifications:
Sensor duty cycles, scheduling constraints, data gaps, and operational conflicts are not part of the model.

- Methodology

Target Object (SGP4)

|

v

Orbit Propagation

|

v

Relative Geometry Calculation

|

v

FOV Crossing Check

|

v

Illumination + Range Check

|

v

Event Detection

Algorithm: Crossing and Visibility Detection Algorithm

Inputs

- Target object TLE → propagate using SGP4 (TEME output)

- Analysis window: [t0,t0+24 hr] where t0 ​= 2025-09-01 00:00:00 UTC

- Time step: Δt (like10 s)

- Space-based tracker orbital elements at epoch: a,e,i,Ω,ω,M0

- Sensor model:

  - FOV = 30° (half-angle = 15°)

  - Boresight = tracker velocity direction

  - Detectable if: crossing AND sunlit AND range < 1000 km

Outputs

- Crossing events: start time, end time

- Visible/detectable events: start time, end time

Step-by-step

- Initialize

  - Read TLE → create SGP4 propagator for the target.

  - Set epoch t0 and build time grid tk=t0+kΔt, for k=0…N over 24 hours.

  - Define constants: Earth radius, μ, FOV half-angle (15°), range threshold (1000 km).

- Loop over time grid (for each tk​)

- Propagate target with SGP4 → robj(tk),vobj(tk) (TEME).

- Propagate tracker with 2-body Kepler model → rtrk(tk),vtrk(tk).

- Compute line-of-sight vector and range:

ρ(tK)=robj(tK)-rtrk(tK),d(tK)=∥ρ(tK)∥

- Compute boresight unit vector:

btK=vtrktKvtrktK

- Compute angle between LOS and boresight:

θtK=cos-1ρtK.btKρ(tK)

- Crossing condition:

CrosstK=θtK≤15°

- Sunlit test:

  - Compute Earth→Sun direction at tK

  - Apply cylindrical eclipse model → Sunlit(tK)∈{True,False}

- Visible/detectable condition:

VisibletK=crosstK∧ SunlittK∧[dtK≤1000km]

- Event extraction

- Convert boolean series crosstK into time intervals by detecting transitions:

  - False→True = start time

  - True→False = end time

- Repeat for VisibletK

- Store each event as (start_time, end_time).

- Print results

- Print all Crossing intervals.

- Print all Visible intervals.
(If no intervals occur, print “None”.)

- Results and Discussion

The developed Python script was executed for a 24-hour analysis window beginning at 2025-09-01 00:00:00 UTC, with a time discretization interval of 10 seconds. During the simulation, the target object orbit was propagated using the SGP4 model based on the provided TLE parameters, while the tracker orbit was propagated using a two-body Keplerian model derived from the specified orbital elements.

For each time step, the algorithm evaluated the relative geometry between the tracker and the target object. The line-of-sight vector was computed, and the angular separation between this vector and the sensor boresight direction (aligned with the tracker velocity vector) was determined. A crossing event was identified when the angular separation fell within the camera’s field-of-view half-angle of 15 degrees, corresponding to a full field-of-view of 30 degrees.

To determine whether the object was detectable, additional visibility constraints were applied. Specifically, the object had to be illuminated by the Sun and lie within a maximum range of 1000 km from the tracker during the crossing interval. Illumination was evaluated using a simplified cylindrical Earth shadow model, which determines whether the object lies within the Earth's shadow region.

The results of the simulation indicated that no crossing events occurred during the 24-hour analysis period. Consequently, no visibility or detectable events were identified. This outcome suggests that the relative orbital geometry between the tracker and the target object did not produce any instances in which the object entered the camera's field of view within the analyzed time window.

Although no events were detected in this particular scenario, the simulation framework successfully demonstrated the capability to evaluate geometric sensor-object relationships, illumination conditions, and range constraints in a consistent computational workflow. The approach also highlights how event detection can be performed efficiently using discretized propagation combined with boolean time-series analysis to extract event intervals.

The absence of crossings in the simulated window is not unexpected in orbital analysis. Even when two objects share similar orbital inclinations, their relative phasing and right ascension of ascending node can prevent line-of-sight alignment for extended periods. In operational SSA systems, such analyses are routinely repeated over longer time horizons to identify potential observation opportunities.

- Scalable Architecture for Large-Scale Analysis

## Design goals

- Throughput: millions of object–tracker checks/day

- Avoid brute force O(Nobj Ntrk Nt)

- Two-stage filtering: cheap geometry → expensive visibility

- Batch + vectorize computations (NumPy arrays)

- Safe parallelization across trackers and/or object batches

- Compact outputs: event intervals, not full time series

## 1) High-level program architecture (modules)

- Catalog: Parse ~30k TLEs → Satrec list + metadata (NORAD, epoch, mean motion).

- Time Grid Builder

  - Coarse grid over 24 h: Δtcoarse​

  - Local refined grid: Δtfine​  only near candidates.

- Propagation

  - Objects: batched SGP4 propagation (sgp4_array if available).

  - Trackers: propagate each tracker on coarse grid (2-body or SGP4).

- Coarse Candidate Scan: Fast range + cone/FOV gating to generate sparse candidates.

- Window Builder: Merge candidate hits into time windows per (tracker, object).

- Refined Event Finder: Recompute in each window at fine step; extract start/end of crossing.

- Visibility Filter: Apply sunlit + range threshold only within crossing windows.

- Event Store / Reporting: Store intervals: (tracker_id, object_id, type, start, end, min_range, etc.) in Parquet/CSV.

## 2) Core efficiency strategies

### A) Two-stage time stepping

- Stage 1 (coarse): scan all objects with Δtcoarse​ to detect possible events.

- Stage 2 (fine):  get event start/end times by running Δtfine​ only inside candidate windows.

This process reduces the number of costly computations by several magnitudes.

### B) Early geometric gating (cheap → expensive)

For each tracker at time tk​:

- Compute LOS: ρ(tK)=robj-rtrk, d(tK)=∥ρ∥

- Compute cone angle: θtK=cos-1ρ∙v

- Candidate if: d≤dmax+Δd ∧ θ≤θhalf+Δθ

Use margins (like  +50 km, +1–2°) to prevent overlooking boundary events that are due to coarse dt.

### C) Batch + vectorize

- Processing of objects is done in batches (2,000–5,000) with NumPy arrays

- LOS, range, and cone-angle calculations are vectorized

- Eliminate Python loops inside the mathematics; maintain the innermost loop in NumPy.

### D) Parallelization (safe & simple)

- Trackers are parallelized (up to 100)

- Within each tracker: process objects in batches (vectorized).

- Implementation: ProcessPoolExecutor (trackers) + vectorization (batches).

### E) Cache shared computations

- The grid of sun direction vs time is common → calculate once every time grid and reuse.

- Cache tracker state on grid: rtrk[k], vtrk[k]

3) Pipeline flowchart

Load TLE catalog (30k) + Trackers (≤100)

|

Build coarse time grid (dt_coarse) + Sun cache

|

Parallel for each tracker

|

Propagate tracker state on grid (r_tr[k], vhat[k])

|

COARSE CANDIDATE SCAN (batched objects):

SGP4 batch -> range gate + cone gate -> candidate hits (obj_id, k)

|

Merge hits -> candidate windows per object

|

REFINED SCAN within windows (dt_fine):

recompute crossing mask -> crossing intervals

|

VISIBILITY FILTER:

sunlit + range<1000 km within crossing intervals -> visible intervals

|

Write EventStore (intervals) + summary stats

## 4) Complexity

### Naive brute force (too big)

Example with 10 s step: 30,000×100×8,640≈25.9 billion checks/day.                           Not practical.

### Optimized approach: With coarse scan + candidate windows

- Coarse grid at 60 s → 1440 steps/day (6× fewer samples)

- Candidate gating reduces the object set per tracker per step dramatically

- Fine scan runs only for a small fraction of (tracker, object) pairs

Result: computation reduced to millions of checks per day, which is tractable on modern multi-core systems.

5) Pseudo-code (compact)

load catalog, trackers

build coarse time grid

precompute sun vectors

parallel for each tracker:

propagate tracker on grid

candidates = coarse_geometry_scan(objects)

windows = merge_candidate_times(candidates)

crossings = refined_scan(windows)

visible = apply_visibility(crossings)

store events

- Conclusion

This project presented a computational approach for identifying crossing and visibility events between a tracking sensor and a space object using orbital propagation and geometric analysis. The framework combines the SGP4 propagator for TLE-based orbit prediction with a simplified two-body model for sensor orbit propagation. Sensor detection conditions were modeled through field-of-view constraints, range thresholds, and basic illumination checks.

The implemented algorithm successfully demonstrated how relative geometry between a tracker and a space object can be evaluated over a defined time window to identify potential observation opportunities. Although no crossings or detectable events were found during the analyzed 24-hour period, the methodology proved effective for performing systematic sensor-object screening.

Beyond the single-object scenario, the study also proposed a scalable program architecture capable of supporting large-scale screening problems involving tens of thousands of space objects and multiple sensors. By incorporating techniques such as coarse-to-fine time stepping, vectorized computations, geometric pre-filtering, and parallel processing, the system can significantly reduce computational cost while maintaining accuracy in event detection.

Such scalable approaches are essential for modern space surveillance and tracking applications, where large object catalogs must be processed continuously to support space safety and operational decision-making. The presented framework therefore provides a foundation for extending the analysis to larger object populations and more complex sensor networks in future work.